# Power transformer specification for PMP20550		10/17/2016

Program:

TI Field Contact:

Electrical:

## Circuit Topology:	Primary-side SEPIC, with Isolated Flyback

Frequency:		MHz sync, 850KHz min free running, 1020KHz max

Primary Voltage:	Min: 32 Vdc		Nom: 48 Vdc		Max: 57 Vdc

Secondaries:		Output1:   12Vdc @ 1Adc max	(isolated secondary)

Output2:   9Vdc @ 0.02Adc max	(primary referenced bias)

## Volt-S Product:	19 V-uS max (850 KHz min, DF max = 0.5, Vin min = 32V)

Primary Inductance:	45uH +/- 10%, 1.05Apk, 0.6Arms (at Vin = 32V)

Leakage Inductance:	1% Desired (low leakage will be critical for proper operation)

Turns Ratio:		(PRI : Sec1: Sec2)	2.5 : 1 : 0.75

Current Rating:	Primary:	0.6 Arms, 1.05Apk,	0.25 ohms desired

## Sec1:		1.50 Arms, 2.05 Apk,	0.05 ohms desired

## Sec2:		0.030 Arms, 0.041 Apk

Isolation:		1500V or typical

Shielding:		No

Insulation Resistance: Unknown

Certifications:		Unknown

Environment:

Operating Temperature Range:	Ambient -40 oC to +85 oC

Temperature Rise:			40 o C max hot spot

Airflow: 				No

Physical:

Dimensions:		Surface mount, high efficiency first, then size, then cost

Maximum Height:  	TBDmm Max Height